CMPG 325 (2026) — Computer Networks

Milestone 1 — Client Design Review

Section 4: IP Addressing Plan

| Student | Choshane, KBP |
| --- | --- |
| Student Number | 37774425 |
| Project ID | CMPG325-2026-010 |
| Client ID | CLI-010 |
| Assigned Addressing Block | 10.15.0.0/16 |

# 1. Addressing Approach

The client has assigned the address block 10.15.0.0/16 for this project. This plan divides that block into seven fixed-size /24 subnets — one per VLAN defined in the Client Requirements document (Section 1 of this Milestone 1 submission), plus one /24 reserved for switch/native management traffic (VLAN 99).

Fixed /24 subnetting was chosen over variable-length subnet masking (VLSM) for the following reasons:

- The assigned /16 provides far more address space (65,536 addresses) than this branch requires, so address-space efficiency is not a binding constraint.

- Uniform /24 subnets are simple to explain and verify during the video demonstration, reducing the risk of configuration or calculation errors.

- The third octet of each subnet has been deliberately matched to its VLAN ID (e.g. VLAN 20 -> 10.15.20.0/24), making the addressing scheme self-documenting.

- Room remains within the block for additional departments or subnets to be added later without redesigning the scheme.

# 2. VLAN Subnet Allocation

| VLAN | Department | Subnet (CIDR) | Usable Host Range | Broadcast | Gateway (Router) |
| --- | --- | --- | --- | --- | --- |
| 10 | Management/Admin | 10.15.10.0/24 | 10.15.10.1 - 10.15.10.254 | 10.15.10.255 | 10.15.10.1 |
| 20 | Finance (isolated) | 10.15.20.0/24 | 10.15.20.1 - 10.15.20.254 | 10.15.20.255 | 10.15.20.1 |
| 30 | Tellers/Customer Service | 10.15.30.0/24 | 10.15.30.1 - 10.15.30.254 | 10.15.30.255 | 10.15.30.1 |
| 40 | Loans | 10.15.40.0/24 | 10.15.40.1 - 10.15.40.254 | 10.15.40.255 | 10.15.40.1 |
| 50 | IT/Servers | 10.15.50.0/24 | 10.15.50.1 - 10.15.50.254 | 10.15.50.255 | 10.15.50.1 |
| 60 | Guest Wi-Fi | 10.15.60.0/24 | 10.15.60.1 - 10.15.60.254 | 10.15.60.255 | 10.15.60.1 |
| 99 | Native/Switch Management | 10.15.99.0/24 | 10.15.99.1 - 10.15.99.254 | 10.15.99.255 | 10.15.99.1 |

# 3. Address Assignment Method

Not all devices are addressed the same way. Static addressing is used where a fixed, predictable address is operationally required (gateways, servers, switch management, and the isolated Finance segment); DHCP is used for general staff end devices to simplify management.

| Device | VLAN | Assignment Method | Example / Notes |
| --- | --- | --- | --- |
| Router subinterfaces | All (10,20,30,40,50,60,99) | Static | Acts as default gateway for each VLAN (router-on-a-stick) |
| Switch management (SVI) | 99 (Native) | Static | 10.15.99.2 / .3 per switch, for remote SSH management |
| IT/Server (DHCP + file server) | 50 | Static | 10.15.50.10 |
| Finance PC(s) | 20 | Static (recommended) | 10.15.20.10 upward - avoids reliance on DHCP for the isolated segment |
| Management, Tellers, Loans, Guest PCs | 10 / 30 / 40 / 60 | DHCP | Leased from DHCP server in VLAN 50, scoped per VLAN |

# 4. Relationship to Finance Isolation and CR9

The Finance VLAN (20) is addressed identically in structure to every other VLAN, since IP addressing alone does not enforce isolation — isolation is enforced through VLAN segmentation and inter-VLAN access control, as described in the Client Requirements document. Static addressing is recommended for Finance specifically so that any access control rules applied at the router can reference fixed, known host addresses rather than a changing DHCP pool.

The off-site administrator covered by CR9 is not assigned an address from 10.15.0.0/16, since they connect from outside the branch over a simulated WAN/Internet link. They will reach the router's outward-facing interface and, from there, use SSH to manage devices addressed within VLAN 99 (switch management) and the router itself.

# 5. Notes for Implementation (Milestone 2 onward)

- Router subinterfaces will be configured with 802.1Q trunking, one subinterface per VLAN, each holding the .1 gateway address shown above.

- DHCP scopes for VLANs 10, 30, 40, and 60 will exclude the .1-.9 range to reserve it for infrastructure and static assignments.

- This plan will be revisited if any addressing changes are required once configuration begins; any changes will be reflected here first, with all diagrams and configuration updated to match.