CMPG 325 — Computer Networks

Milestone 2 — Full Technical Documentation & Video Presentation Guide

| Student | Choshane, KBP (37774425) |
| --- | --- |
| Project ID / Client | CMPG325-2026-010 / CLI-010 — Bokamoso Savings & Credit Union (Potchefstroom) |
| Milestone | Milestone 2 — Client Implementation Review |
| Assigned technical challenge | VLANs (switch-based segmentation design) |
| Design constraint | Finance department isolated from all general users |
| Change request | CR9 — one off-site administrator requires secure remote management access |

# 1. Overview

This document consolidates everything implemented, configured, and tested for Milestone 2 (Client Implementation Review), and doubles as a script for the required 15–20 minute individual video demonstration. It covers the final topology, the VLAN design and why it satisfies the assigned technical challenge, every stage of configuration performed, all testing evidence, and a structured presentation guide mapped directly to the video requirements in the project brief.

# 2. Final Topology

The topology builds directly on the Milestone 1 Physical and Logical Topology diagrams, expanded during implementation to reflect a more realistic device count and a genuinely wireless off-site connection. Two deliberate changes were made from the original Milestone 1 diagrams, both explained fully in Section 6 (Troubleshooting): the Cloud-PT WAN device was replaced with a plain switch, and a wireless Access Point was added for the off-site administrator's connection. Neither change affects the VLAN design, the Finance isolation constraint, or CR9 — both remain fully and correctly satisfied.

## 2.1 Devices

| Device | Model | Role |
| --- | --- | --- |
| R1 | Cisco 2911 | Router-on-a-stick: inter-VLAN routing via subinterfaces on Gi0/0 and Gi0/1; Gi0/2 is the WAN-facing interface for CR9 |
| SW-A | Cisco 2960 | Access switch for VLANs 10, 20, 99. Gi0/1 trunks to R1 |
| SwitchB | Cisco 2960 | Access switch for VLANs 30, 40, 50, 60, 99. Gi0/1 trunks to R1 |
| WAN | Cisco 2960 (unconfigured) | Plain Layer 2 bridge representing the ISP/Internet path between R1 and the off-site administrator |
| AP-Remote | Access Point (AP-PT) | Provides the wireless “last hop” for the off-site administrator, representing their own local wireless setup (e.g. home Wi-Fi), SSID Bokamoso-Remote, WPA2-PSK |
| Laptop-OffsiteAdmin | Laptop-PT + WPC300N wireless module | The off-site administrator's device; connects wirelessly to AP-Remote, static IP 203.0.113.2/30 |

# 3. VLAN Design — the Assigned Technical Challenge

VLANs (switch-based segmentation) are the specific networking challenge assigned to this project (Section 9 of the brief, Intermediate difficulty). A VLAN (Virtual LAN) allows a single set of physical switches to be logically divided into multiple separate broadcast domains — devices in different VLANs cannot see each other's broadcast traffic and, without explicit routing, cannot reach each other at all, even if physically plugged into the same switch. This is what allows one pair of switches to safely serve six different departments (plus a management VLAN) instead of requiring six sets of physical switches.

Seven VLANs were configured, matching the Milestone 1 IP Addressing Plan exactly:

| VLAN | Department | Subnet | Gateway | Devices |
| --- | --- | --- | --- | --- |
| 10 | Management | 10.15.10.0/24 | 10.15.10.1 | PC-Mgmt (.10), PC-Mgmt2 (.11), PC-Mgmt3 (.12) — all DHCP |
| 20 | Finance (ISOLATED) | 10.15.20.0/24 | 10.15.20.1 | PC-Finance (.10), PC-Finance2 (.11), PC-Finance3 (.12) — all static |
| 30 | Tellers | 10.15.30.0/24 | 10.15.30.1 | PC-Teller (.10), PC-Teller2 (.11) — both DHCP |
| 40 | Loans | 10.15.40.0/24 | 10.15.40.1 | PC-Loans (.10) — DHCP |
| 50 | IT/Servers | 10.15.50.0/24 | 10.15.50.1 | Server-IT (.10) — static |
| 60 | Guest | 10.15.60.0/24 | 10.15.60.1 | PC-Guest (.10) — DHCP |
| 99 | Native/Mgmt | 10.15.99.0/25 (SW-A) + .128/25 (SwitchB) | 10.15.99.1 / .129 | Switch management SVIs; carried on both trunks |

VLAN 99 required one adjustment during implementation: because SW-A and SwitchB are not directly connected to each other (only to the router), their native/management VLAN segments are physically separate and cannot share one identical subnet on the router. The originally planned 10.15.99.0/24 was therefore split into two /25 subnets — still entirely within the block allocated in the Milestone 1 IP Addressing Plan, just subdivided to reflect the real topology.

# 4. Configuration Performed

## 4.1 VLAN creation and access port assignment

On each switch, VLANs were created and named, then each access port (the ports connecting to end devices) was assigned to its correct VLAN using switchport mode access and switchport access vlan <id>. Spanning-tree portfast was enabled on every access port so that end devices could obtain a DHCP address immediately rather than waiting through the default ~30–50 second STP listening/learning delay.

## 4.2 Trunking

The link between each switch and the router (Gi0/1 on both SW-A and SwitchB) was configured as an 802.1Q trunk, carrying every VLAN present on that switch plus the native VLAN (99). This allows tagged traffic from multiple VLANs to travel over a single physical cable to the router.

## 4.3 Inter-VLAN routing (router-on-a-stick)

R1 has only two physical interfaces facing the switches (Gi0/0 and Gi0/1), but needed to act as the default gateway for seven separate VLANs. Each physical interface was divided into subinterfaces (e.g. Gi0/0.10, Gi0/0.20), each tagged to one VLAN via encapsulation dot1Q and given the gateway IP address from the addressing plan. This is the standard “router-on-a-stick” design: one physical link, multiple virtual routers, one per VLAN.

## 4.4 Addressing — static and DHCP

Server-IT and every Finance PC were assigned static addresses, matching the Milestone 1 Addressing Plan's approach of keeping the isolated Finance segment and the shared server on fixed, predictable addresses. All other end devices (Management, Tellers, Loans, Guest) obtain addresses automatically from DHCP pools configured directly on R1, one pool per VLAN, each excluding the .1–.9 range reserved for infrastructure.

## 4.5 Finance isolation (the design constraint)

An extended access control list, BLOCK_FINANCE, was created to enforce the brief's constraint that Finance must be isolated from all general users. The ACL denies traffic from the Management, Tellers, Loans, and Guest subnets toward the Finance subnet, with a final permit ip any any statement so that Finance's own legitimate traffic (and traffic to/from the IT/Servers VLAN, deliberately left unblocked for scenarios such as backups) is not affected. The ACL is applied outbound on R1's Finance subinterface (Gi0/0.20), so that it filters traffic as it is delivered toward Finance rather than traffic leaving Finance.

## 4.6 CR9 — secure remote management access

To satisfy CR9 without expanding its scope, one local administrator account was created on R1 (username admin), SSH version 2 was enabled, RSA keys were generated, and the router's VTY lines were restricted to accept SSH connections only (transport input ssh), with Telnet fully disabled. This means any remote management session is encrypted end-to-end — unlike Telnet, which sends credentials in plain text.

To make the “off-site” nature of CR9 concrete rather than just conceptual, the administrator's PC connects to the network wirelessly: it associates with an Access Point over WPA2-PSK-secured Wi-Fi, which itself connects back into the network through a separate WAN path (entirely distinct from the branch's internal VLAN trunks). This mirrors how any real remote connection works — a wireless “last hop” to the user's own local network, riding on wired infrastructure the user never directly manages beyond that point.

# 5. Testing and Verification

Each requirement was tested from the perspective of an actual end device, not just checked in the router's configuration, so that the evidence reflects genuine behaviour rather than intended behaviour.

| Test | Expected result | Actual result |
| --- | --- | --- |
| PC-Mgmt → ping PC-Finance (10.15.20.10) | Blocked | Blocked — confirmed |
| PC-Mgmt → ping own gateway (10.15.10.1) | Success (control) | Success — confirmed |
| PC-Mgmt2 → ping PC-Finance2 (10.15.20.11, added post-expansion) | Blocked | Blocked — confirmed |
| PC-Finance2 → ping PC-Finance3 (10.15.20.12) | Success (same VLAN) | Success — confirmed |
| PC-Mgmt/Teller/Loans/Guest → DHCP request | Successful lease from correct VLAN pool | All successful after portfast fix |
| Off-site laptop → ping R1 WAN interface (203.0.113.1) | Success | Success (wireless, 11–29ms) |
| Off-site laptop → SSH into R1 (10.15.99.1) | Success, Telnet rejected | Success — CR9 demonstrated over wireless |

# 6. Troubleshooting Summary

Three issues were encountered and resolved during implementation; each is documented in full in troubleshooting-notes.md in the evidence folder. In brief:

- Cloud-PT's DSL configuration only supports pairing a Modem port with an Ethernet port, not two Ethernet ports directly — the wrong mechanism for a simple point-to-point WAN link. Resolved by replacing the Cloud with a plain, unconfigured switch, which bridges connected ports automatically.

- Packet Tracer's default Laptop-PT wireless configuration screen requires a specific wireless module (WPC300N for laptops); the default wired module produced an error. Resolved by swapping the module while the device was powered off, then matching SSID and WPA2-PSK settings between the laptop and the Access Point.

- The Finance ACL initially had no effect because it was applied in the in direction on Gi0/0.20, which filters traffic leaving Finance rather than traffic destined toward it. Resolved by reapplying the ACL in the out direction, confirmed by retesting.

# 7. Video Presentation Guide

The brief requires a 15–20 minute individual video with an inset webcam view, covering seven specific elements. Suggested structure and talking points below, timed for a ~18 minute video:

## 7.1 Introduce yourself and the assigned client/project (~1 min)

- Name, student number, module, and that this is an individual project.

- Client: Bokamoso Savings & Credit Union, Potchefstroom — a small credit union branch, Banking & Finance industry.

- Assigned technical challenge: VLANs (switch-based segmentation), Intermediate difficulty.

## 7.2 Explain client requirements and network design (~3 min)

- Walk through the Client Requirements document: connectivity, VLAN segmentation, Finance isolation, CR9.

- Show the Logical Topology diagram and explain the department/VLAN structure.

- Explain the addressing approach: 10.15.0.0/16 assigned block, subnetted into /24s per VLAN, with the reasoning (simplicity, room for scalability/future branches) and the one exception for VLAN 99 (split into two /25s due to the physical topology).

## 7.3 Demonstrate the Packet Tracer implementation (~3 min)

- Show the full built topology, pointing out the router, both access switches, the WAN switch, the Access Point, and all end devices.

- Mention the expansion beyond the original 1-per-VLAN design (3 Mgmt, 3 Finance, 2 Tellers) — explain this was done to test the design under more realistic conditions, and that the ACL and DHCP scopes required no changes to accommodate it.

## 7.4 Demonstrate the assigned networking challenge — VLANs (~4 min)

- Show show vlan brief and show interfaces trunk on both switches — explain what each column means.

- Show show ip interface brief on R1 — explain router-on-a-stick and subinterfaces.

- Explain why VLANs were the right fit: department-level segmentation without needing separate physical switches per department, and forming the basis for enforcing Finance isolation.

## 7.5 Show relevant configuration and connectivity/testing (~4 min)

- Show the BLOCK_FINANCE ACL configuration and explain the in vs out direction lesson learned.

- Live-demonstrate: ping from Mgmt to Finance (fails), ping from Mgmt to its own gateway (succeeds), ping between two Finance PCs (succeeds).

- Live-demonstrate CR9: from the off-site laptop, show the Wi-Fi connection to the Access Point, then ping the router's WAN interface, then SSH into R1, showing the login banner and successful session.

## 7.6 Discuss important design decisions and troubleshooting (~2 min)

- Explain the Cloud → switch replacement and why it doesn't compromise the design.

- Explain the wireless module swap for the off-site laptop.

- Explain the ACL direction fix — a good moment to show genuine understanding rather than just correct output.

## 7.7 Reflection (~1 min)

- What you learned: router-on-a-stick, ACL direction, VLAN segmentation in practice versus theory.

- What you'd do differently or extend given more time (e.g. VLSM for tighter address efficiency, redundant links, more granular ACL rules).

# 8. Files Referenced in This Document

- Client_Requirements.docx, IP_Addressing_Plan.docx, Physical_Topology.png, Logical_Topology.png (Milestone 1, in design/)

- R1_running-config.txt, SW-A_running-config.txt, SwitchB_running-config.txt (in config/)

- troubleshooting-notes.md and all testing screenshots (in evidence/)

- Bokamoso_CMPG325_2026010.pkt (final working Packet Tracer file)